Technical Parameters

Measurements

Build Volume	300 x 300 x 330 mm | 11.8 x 11.8 x 12.99 in

Filament diameter	1.75 mm

Layer height	0.05-0.30 mm

Printer dimensions	21,9 kg, 469×521×635 mm; 18.5×20.5×25 in (W×D×H)

Electronics

Mainboard	Custom 32-bit xBuddy electronics with STM32

Stepper Drivers	Trinamic 2130

Stepper motors	Precise 0.9° X,Y stepper motors (prevents VFA)

Extruder

Extruder	Nextruder, Direct Drive, E3D V6 compatible (with adapter)

Extruder electronics	Custom breakout board for connecting fans, thermistors and other parts.

Filament path	Aluminum heatsink, all-metal hotend

Drive system	Nextruder Planetary Gear System, 10:1 gearbox ratio

Cooling fan	High-performance turbine, 360° cooling system

Nozzle	High-flow Prusa Nozzle brass CHT - 0.4 mm + Abrasive-Resistant 0.4mm nozzle in the box

Max. operating temperature

Max nozzle temperature	290 °C / 554 °F

Max heatbed temperature	120 °C / 248 °F

Max chamber temperature	60 °C / 140 °F

Controls

LCD screen	3.5″ graphic 65k color screen

Touch control	Yes

Firmware update interface	USB drive, Prusa Connect, Prusa Mobile App

Prusa Connect support	Yes

Prusa App for iOS and Android support	Yes

Quick-Swap nozzle system	Yes

Lighting	Yes

Accessories

Internal camera	Included in the box, installation optional

MMU3 support	Yes. MMU3 (available Jan 2026)

Advanced Filtration System	optional addon

GPIO Board compatibility	Yes

Accelerometer compatibility	Yes, built-in

Connectivity

Ethernet connection	 Yes

Other connectivity	NFC receiver

Sensors

Filament Sensor	Yes (2 sensors)

Loadcell sensor	Yes

Thermistors	5 high-precision thermistors (genuine Semitec)

Fan motors monitoring	Yes

Door sensor	Yes

Printing

Print medium	USB drive / LAN / internet via Prusa Connect or mobile Prusa App

Print surface	Magnetic heatbed with removable PEI spring steel sheets

First layer calibration	Fully automatic (thanks to the Load Cell sensor)

Bed calibration	Automatic, Mesh Bed Leveling (only on print area)

Input shaper	Yes

Supported materials	PLA, PETG, Flex, PVA, PC, PP, CPE, PVB and when using the optional Advanced Filtration System ABS, ASA, HIPS, PA

Phase Stepping	Yes